CODE FOR IMPLEMENTATION

"""

NanoSense Core Architecture Simulator

Models Booth's Multiplication, Restoring Division, IEEE-754 Addition,

Pipeline Hazards, and Cache Access for ultra-low-power IoT nodes.

"""

# ==========================================

# TASK 1: Arithmetic Logic Unit (ALU)

# ==========================================

def arithmetic_shift_right(val, bits):

"""Helper for Booth's Algorithm"""

sign_bit = val & (1 << (bits - 1))

return (val >> 1) | sign_bit

def booths_multiplier_8bit(multiplicand, multiplier):

"""Simulates an 8-bit signed Booth's Multiplier"""

print(f"--- Booth's Multiplication: {multiplicand} * {multiplier} ---")

# Convert to 8-bit two's complement integers

M = multiplicand & 0xFF

Q = multiplier & 0xFF

A = 0x00

Q_minus_1 = 0

for cycle in range(8):

Q_0 = Q & 1

# Check Booth's conditions (Q_0, Q_minus_1)

if Q_0 == 1 and Q_minus_1 == 0:

A = (A - M) & 0xFF  # Subtract M

elif Q_0 == 0 and Q_minus_1 == 1:

A = (A + M) & 0xFF  # Add M

# Arithmetic Shift Right [A, Q, Q_minus_1]

combined = (A << 9) | (Q << 1) | Q_minus_1

sign_bit = combined & (1 << 17)

combined = (combined >> 1) | sign_bit

A = (combined >> 9) & 0xFF

Q = (combined >> 1) & 0xFF

Q_minus_1 = combined & 1

result = (A << 8) | Q

# Convert 16-bit 2's complement to signed integer

if result & (1 << 15):

result -= (1 << 16)

print(f"Result: {result} (Cycles: 8)\n")

return result

def restoring_divider_8bit(dividend, divisor):

"""Simulates an 8-bit unsigned Restoring Divider"""

print(f"--- Restoring Division: {dividend} / {divisor} ---")

if divisor == 0:

raise ZeroDivisionError("Hardware fault: Divide by zero")

A = 0

Q = dividend & 0xFF

M = divisor & 0xFF

for cycle in range(8):

# Shift Left [A, Q]

A = ((A << 1) | ((Q >> 7) & 1)) & 0xFF

Q = (Q << 1) & 0xFF

# Subtract Divisor

A = (A - M) & 0xFF

if A & 0x80: # If A is negative (MSB is 1)

Q = Q & ~1 # Set Q[0] = 0

A = (A + M) & 0xFF # Restore A

else:

Q = Q | 1 # Set Q[0] = 1

print(f"Quotient: {Q}, Remainder: {A} (Cycles: 8)\n")

return Q, A

# ==========================================

# TASK 2: IEEE-754 Floating Point Addition

# ==========================================

import struct

def float_to_bin(f):

"""Helper to get 32-bit integer representation of float"""

return struct.unpack('>I', struct.pack('>f', f))[0]

def ieee754_add(f1, f2):

"""Models hardware alignment and normalization for float addition"""

print(f"--- IEEE-754 Float Addition: {f1} + {f2} ---")

b1, b2 = float_to_bin(f1), float_to_bin(f2)

# Extract fields (assuming positive numbers for simplicity in this model)

exp1 = (b1 >> 23) & 0xFF

exp2 = (b2 >> 23) & 0xFF

mant1 = (b1 & 0x7FFFFF) | 0x800000 # Add hidden bit

mant2 = (b2 & 0x7FFFFF) | 0x800000

# Align Exponents

if exp1 > exp2:

shift = exp1 - exp2

mant2 >>= shift

final_exp = exp1

else:

shift = exp2 - exp1

mant1 >>= shift

final_exp = exp2

# Add Mantissas

result_mant = mant1 + mant2

# Normalize

if result_mant & 0x1000000: # Overflow in mantissa (carried out of 24th bit)

result_mant >>= 1

final_exp += 1

# Check for Exponent Overflow

if final_exp >= 255:

print("Hardware Exception: Float Overflow! Clamping to Infinity.\n")

return float('inf')

result_mant &= 0x7FFFFF # Remove hidden bit

result_bin = (final_exp << 23) | result_mant

result_float = struct.unpack('>f', struct.pack('>I', result_bin))[0]

print(f"Hardware Result: {result_float}\n")

return result_float

# ==========================================

# TASK 3: Pipeline Hazard Simulation

# ==========================================

def evaluate_pipeline_hazards():

"""Models cycle counts for sensor fusion sequence with and without forwarding"""

print("--- 5-Stage Pipeline Simulation ---")

print("Sequence: LW R1 -> ADD R3, R1 -> SW R3")

base_cycles = 5 # Time for first instruction to traverse IF-ID-EX-MEM-WB

num_instructions = 3

# 1. Without Forwarding: Data hazard on R1 requires 2 stall cycles.

# LW finishes MEM at cycle 4, writes at 5. ADD needs R1 at ID (cycle 3).

stalls_without = 2

total_cycles_no_fwd = (base_cycles - 1) + num_instructions + stalls_without

# 2. With EX/MEM Forwarding:

# LW data ready after MEM (cycle 4). ADD needs it at EX (cycle 4). 1 load-use stall.

stalls_with = 1

total_cycles_fwd = (base_cycles - 1) + num_instructions + stalls_with

print(f"Cycles without forwarding: {total_cycles_no_fwd}")

print(f"Cycles with EX/MEM forwarding: {total_cycles_fwd}")

print(f"Power Savings: {(total_cycles_no_fwd - total_cycles_fwd) / total_cycles_no_fwd * 100:.1f}%\n")

# ==========================================

# TASK 4: Cache vs. Scratchpad (TCM)

# ==========================================

class DirectMappedCache:

"""Simulates a direct-mapped cache for the circular sensor buffer"""

def __init__(self, size_bytes=16, block_bytes=4):

self.num_blocks = size_bytes // block_bytes

self.block_size = block_bytes

self.cache_tags = [None] * self.num_blocks

self.hits = 0

self.misses = 0

def access(self, address):

block_address = address // self.block_size

index = block_address % self.num_blocks

tag = block_address // self.num_blocks

if self.cache_tags[index] == tag:

self.hits += 1

else:

self.misses += 1

self.cache_tags[index] = tag # Fetch block into cache

def evaluate_memory_strategy():

print("--- Memory Strategy Simulation ---")

# Simulating fetching 32-bit (4-byte) floats sequentially from a circular buffer

access_trace = [0, 4, 8, 12, 16, 20, 24, 28]

# 1. Using Standard Data Cache

d_cache = DirectMappedCache(size_bytes=16, block_bytes=4)

for addr in access_trace:

d_cache.access(addr)

total_accesses = len(access_trace)

miss_rate = (d_cache.misses / total_accesses) * 100

print(f"Standard D-Cache Miss Rate: {miss_rate}% (Compulsory misses on streaming data)")

print("Conclusion: D-Cache tag comparisons waste power. Use Scratchpad/TCM for 1-cycle guaranteed access.\n")

# ==========================================

# RUN SIMULATIONS

# ==========================================

if __name__ == "__main__":

booths_multiplier_8bit(5, -3)

restoring_divider_8bit(14, 3)

ieee754_add(1.5, 2.75)

# Trigger Edge Case: Overflow

ieee754_add(3.4e38, 3.4e38)

evaluate_pipeline_hazards()

evaluate_memory_strategy()

OUTPUT